# Ahsanullah	University of Science & Technology

## Department of Computer Science & Engineering

Course No	: CSE 2214

Course Title	: Assembly Language Programming Sessional Assignment No	: 05

Date of Submission	: 20th September,2026

Submitted To	:	Ms. Sumaiya Nuha Mustafina & Ms. Tasnuva

Binte Rahman

Submitted By-

Group	: A1

Name	: Sakif Ahmed Rafi Id		00724105101024

Section	: A

### Question:

Write an assembly program to implement the following operations using the seven segment display and LEDs available in the emulation kit.

- Implement a three digit counter that will start from the last three digits of your student ID and ends at 700, increasing by 3 at each step. For example, if the last three digits of your ID is 054; the expected output:

054 -> 057 -> 060.   ..696 -> 699

- Simultaneously, turn ON the LEDs two at a time in a reverse sequential order.

The sequence should be:

LED 7,8 -> LED 5,6 -> LED 3,4 -> LED 1,2 -> LED 7,8 ->........

At any particular time, only two LEDs should be ON. After the first two LEDs are turned ON, the sequence should restart from LED 7 and 8.

### Answer:

;00724105101024

L1:

MOV BX,VAL

MOV TEMP,BX

;2030H

XOR DX,DX

MOV AX,VAL

MOV BX,100

DIV BX

MOV VAL,DX

LEA SI,DIGITS

ADD SI,AX

MOV DX,2030H

MOV AL,[SI]

OUT DX,AL

;2031H

XOR DX,DX

MOV AX,VAL

MOV BX,10

DIV BX

MOV VAL,DX

LEA SI,DIGITS

ADD SI,AX

MOV DX,2031H

MOV AL,[SI]

OUT DX,AL

;2032H

MOV AX,VAL

LEA SI,DIGITS

ADD SI,AX

MOV DX,2032H

MOV AL,[SI]

OUT DX,AL

;LED

LEA SI,LED

ADD SI,LED1

MOV AL,[SI]

MOV DX,2070H

OUT DX,AL

CALL DELAY

DEC LED1

CMP LED1,0

JG SKIP

MOV BX,4

MOV LED1,BX

SKIP:

MOV AL,00000000B

OUT DX,AL

;LOOP CONTROL

MOV BX,TEMP

ADD BX,3

MOV VAL,BX

CMP VAL,700D

JLE L1

VAL DW 24

TEMP DW ?

LED1 DW 4

DIGITS DB 0111111B, 0000110B, 1011011B, 1001111B, 1100110B, 1101101B, 1111101B, 0000111B, 1111111B, 1101111B

LED DB 00000000B, 00000011B, 00001100B, 00110000B, 11000000B

DELAY PROC

MOV CX,50

L5:

LOOP L5

RET

DELAY ENDP