# Ahsanullah	University of Science & Technology

## Department of Computer Science & Engineering

Course No	: CSE 2214

Course Title	: Assembly Language Programming Sessional Assignment No	: 06

Date of Submission	: Thursday, 1st October, 2026

Submitted To	:	Ms. Sumaiya Nuha Mustafina & Ms. Tasnuva

Binte Rahman

Submitted By-

Group	: A1

Name	: Sakif Ahmed Rafi

Id	00724105101024

Section	: A

### Write an assembly language program that displays a sequence formed by interleaving the last two digits of your studentID with the last two letters of your name(digit-letter-digit-letter-...), one by one, on consecutive dot matrix displays. After the sequence has been shown, the program should continuously scroll the pattern from right to left until the entire sequence reaches the leftmost position, then reverse the direction and scroll it from left to right. Repeat the process continuously with suitable delays.

For example, If the Student’s name is Ahmed Abdullah, and his ID is 00724105101027, the sequence to be displayed should be 2A7H

;00724105101024

MOV BX,-1

L1:

LEA SI,ARR

MOV DX,PORT1

MOV CX,20

L2:

MOV AL,[SI]

OUT DX,AL

INC SI

INC DX

LOOP L2

ADD PORT1,BX

CMP PORT1,2000H

JL MOV_RIGHT2

CMP PORT1,2014H

JG MOV_LEFT2

CMP BX,1

JE MOV_RIGHT1

CMP BX,-1

JE MOV_LEFT1

MOV_LEFT1:

MOV DX, PORT1

CALL DELAY

ADD DX, 20

MOV AL, 0000000B

OUT DX, AL

CALL DELAY

JMP L1

MOV_RIGHT1:

CALL DELAY

MOV DX,PORT1

DEC DX

MOV AL,0000000B

OUT DX,AL

CALL DELAY

JMP L1

MOV_LEFT2:

MOV PORT1,2013H

MOV BX,-1

MOV DX,2027H

MOV AL,0000000B

OUT DX,AL

CALL DELAY

JMP L1

MOV_RIGHT2:

MOV PORT1,2001H

MOV BX,1

MOV DX,2000H

MOV AL,0000000B

OUT DX,AL

CALL DELAY

JMP L1

ARR DB 1111001B,1001001B,1001001B,1001001B,1001111B,1111111B,0001001B,0001001B,0000001B,0000001B,0001111B,0001000B,0001000B,0001000B,1111111B,1000001B,1000001B,1111111B,1000001B,1000001B

T DB 0

PORT1 DW 2014H

PORT2 DW 2000H

DELAY PROC

MOV T,20

L5:

DEC T

CMP T,0

JG L5

RET

DELAY ENDP